/----------------------------------------------------------------------------\

|									     |

|		         ---------------------------- 			     |

|		         IRWIN MAGNETIC SYSTEMS, INC.	 		     |

|		         ----------------------------			     |

|									     |

|   File  : IMAGE.DOC                                                        |

|									     |

|   Title : Notes on the IMAGE Program (2.33)                                |

|									     |

|   By 	  : Applications Engineering Dept.                                   |

|									     |

|   Date  : February 8, 1985                                                 |

|									     |

|									     |

\----------------------------------------------------------------------------/

----------------------------

|         Purpose          |--------------------------------------------------

----------------------------

This program provides a verified image backup and restore capability which is

as fast as is possible with the Irwin tape drive. The backup process only

copies allocated disk space and therefore wastes no time on unused disk

space.  Multiple tape backups are allowed to backup disks larger than 10MB.

The restore process features re-direction, which allows the data to be

restored to a different hard disk or to a hard disk that has an altered bad

block map.

----------------------------

|  Overview of Operation   |--------------------------------------------------

----------------------------

Each copy operation consists of a multiple volume copy which may require more

than one tape. The number of tapes required depends upon the actual amount

and location of the data, and not on the total capacity of the disk.

The storage blocks on the disk which are not part of the operating system

itself or in some file, will not be copied to tape during a backup operation.

This is referred to as "compression". Compression provides a reduction in the

amount of time required to copy each volume (tape), making it roughly

proportional to the actual amount of data to be copied.

The disk is conceptually divided into physical sections which are equal in

capacity to one tape. All the data in one of these sections will go onto a

separate tape from the other disk sections.  If no data is in that section of

the disk and in any sections after that section, then no tape is used for

that section.  If there is data on that section of the disk, then it is

compressed onto the "start" of the corresponding tape. Even if two or more

compressed sections would fit onto one tape, it will not be done because a

provision was made for a future kind software which will allow access to

individual files on a backup tape. Allowing for this provision and placing

several sections on one tape would make the program unduely complicated and

slow, so it was not done. Because of this, it is not ALWAYS the case that

the minimum number of tapes will be used.

The operating system (MS-DOS) allocates disk space as close to the start of

the disk as possible, and has no permenent space fragmentation. This means

that basically, MS-DOS keeps it's disk data fairly well compressed onto the

first section of a disk. The later sections of a disk which is not full are

usually completely empty. Thus, usually the IMAGE program will require as few

tapes as possible to store the disk data. In the event that it does not

require the minumum number of tapes, it still takes roughtly the minimum

amount of total time.

For each volume (tape), the IMAGE program will go through at least one "pass"

consisting of two phases: a "copy" phase, and a "verify" phase. During the

copy phase the program copies data from the source device to the object

device. The first copy phase may take up to 8 minutes if an entire tape is to

be written completely. During the verify phase the program reads data from

the object device to verify that it was written correctly during the previous

copy phase. Because the same part of the tape that was written in the copy

phase is verified in the following verify phase, they take the same amount of

time.  If there are any problems with specific blocks of data during the

verify phase, that data will be reprocessed in the next pass during the copy

phase. If the verify phase is successful, no more passes will be necessary.

If more than one pass is necessary, then the second and following passes are

quite short and get shorter and shorter as less and less data requires

recopying.  Seldom will more than two passes be necessary.

Because additional passes are very short, even when they are required, total

time required for a full tape backup of 10 Megabytes is approximately 18

minutes.  This is composed of two 8 minute passes writing and verifying

the complete tape and 2 minutes of initialization and termination overhead.

Less data will require proportionately less time.

----------------------------

| Operational Instructions |--------------------------------------------------

----------------------------

At any time, if a "^C" (control-C) is typed, this will abort the current task

being attempted and return to the main menu. This includes the ability to

abort out of a sub menu.

Whenever a user input is requested, a default answer appears in brackets.  If

a carriage return is given, with no other input, then the default answer is

selected. If the answer given in response to the question is out of range, no

error message is given, but the question is asked once again.

For all the menus, after one menu item is selected, and the menu is returned

to, the new default menu answer may be different from the last default or the

last chosen entry.  Usually, but not always, it will be the item after the

item chosen last.

------------------------------------------------------------------------------

IMAGE is a menu-driven program with the following expert mode main menu:

-IMAGE MAIN MENU-

(1) Change parameters

(2) Image backup to tape

(3) Image restore from tape

(4) Image copy to another disk

(5) End program

Enter option [2]?

If main menu items 2-4 are chosen and eventually are aborted by the user or

otherwise fail, the new default will be the same as the last item chosen.  On

success of items 2-4, the default will be 5.

When the program is done with a copy operation, it indicates whether it was

successful or it was aborted by the user or by the program itself.

In the novice version of IMAGE, the main menu looks like this:

-IMAGE MAIN MENU-

(1) Image backup to tape

(2) Image restore from tape

(3) End program

Enter option [1]?

If main menu items 1 or 2 are chosen and eventually are aborted by the user

or otherwise fail, the new default will be the same as the last item chosen.

On the success of items 1 or 2, the default will be 3.

------------------------------------------------------------------------------

-GLOBAL PARAMETER MENU-

(1) Informative messages displayed

(2) Soft error warning messages displayed

(3) Errors ignored

(4) Source read retries

(5) Object I/O retries

(6) Verify object

(7) Change to novice version

(8) Tape driver parameter menu

(9) Return to main menu

Which parameters would you like to change [1]?

"Should information msgs be displayed (Y/N) [N]?"

This determines whether "Info: " level messages will be displayed.

"Should warning msgs be displayed (Y/N) [N]?"

This determines whether "Warning: " level messages will be displayed.

"Should unrecoverable block I/O errors be ignored (Y/N) [N]"

If the answer is "N" then the program will immediately abort any copy

operation in which it could not read from one of the source blocks.

Otherwise it will indicate at the end of the copy operation which

files were affected.

"How many times should source reads be retried (1-100) [10]?"

This determines how many times to try reading a source block for the

copy operation before giving up on that block. It should be a low

number when attempting backup or restore as quickly as possible

without	much concern for completeness. (A typical circumstance). This

should be a high number when attempting to assure that as much data

as possible will be backed up or restored. (A more unusual

circumstance).

"How many times should object I/O be retried (1-10) [4]?"

This determines how many times to try writing or verifying an object

block for the copy operation before giving up on that block. It

should be a high number when attempting backup or restore onto a

particular tape or disk without much concern for whether the image

will later be readable. (A typical circumstance). This should be a

low when attempting to assure that as much data as possible will be

readable later. (A more unusual circumstance).

"Should object image be verified (Y/N) [Y]?"

This determines whether there will be a verify phase. By avoiding the

verify phase, half of the backup time or approximately one quarter of

the restore time may be avoided.  However, this is not generally

recommended. Especially so for backup operations because tape media

is not quite as dependable as disk media.

"Should this be changed to the novice version (Y/N) [N]?"

This determines if the novice changes are made and put into memory.

------------------------------------------------------------------------------

-TAPE DRIVER PARAMETER MENU-

(1) Tape physical drive number

(2) Tape spares percentage

(3) Tape positioning retries

(4) Tape timeout limit

(5) Tape gap length

(6) Tape I/O overlapped with disk I/O

(7) Return to global parameter menu

Which tape drive parameter would you like to change [1]?

"What tape physical drive number should be used by default (0-3) [1]?"

This determines the first drive number which will be tested by the

program for the existence of a tape drive. Others will be tried if

this one is not a tape drive.

"What percentage of tape should be used for spares (1-99) [1]?"

Determines the size of the area on the tape reserved for redirecting

data meant to go onto bad physical tape blocks. The size of this area

determines the number of blocks which may be bad before the program

will give up on trying to backup to a particular tape. A very low

number is suggested, since there are usually not very many bad

blocks.

"How many times should tape positioning be retried (1-100) [4]?"

This determines how many times the program can "go past" a particular

block without being able to "find" it, before giving up. A low number

(about 4) is suggested.

"What tape timeout count should be used (1-100) [10]?"

This determines the timeout period for reads and writes to the tape

(approximately seconds). Generally it is best not to change this

parameter. (It's there for Irwin Magnetic's internal use).

"What tape gap size should be used (10-100) [23]?"

This determines the gap size in bytes between sectors on the tape.

Changing this may effect the ability to read some tapes, sometimes

positively and sometimes negatively. This depends on a lot of

different variables. It may be worth a try to change this by as much

as 50% if you have a particular tape with a lot of "record not found"

errors, for example.

"Should tape I/O be overlapped with disk I/O (Y/N) [Y]?"

On some machines, tape data may be DMA'ed into memory while

simultaneously accessing the disk.  For this to be advantageous, the

disk I/O must be faster than the tape I/O, in general, or tape will

not be positioned correctly for subsequent I/O operations.  If the

default	for this question is "Y" and backups or restores seem to take

too long because the tape is moving in short back and forth

movements, setting this to "N" may speed things up.  With "N"

selected, as much I/O as possible is done at once on the tape (actual

amount depends on the amount of memory available for buffers), and a

short backspace is done before each disk access. The backspace

assures that the tape is positioned correctly for subsequent tape I/O

operations.

----------------------------

|         Caveats          |--------------------------------------------------

----------------------------

(Note that all of the following are low probability occurrences.)

This design cannot provide for compressing or redirecting the part of the

disk (often zero length) which is not assigned to MS-DOS, but instead

assigned to another operating system. It does save and restore this part if

it is represented in the MS-DOS DPB (which is not likely), but since it is

logically structured in a different way than the MS-DOS part, it cannot be

tampered with. Thus restoration errors in this part can only be recovered by

retry but not redirection. Little specific information can be provided to the

user about files destroyed in this case. It is wise to suggest backing it up

separately using a similar program designed for the other operating system.

File Allocation Units (FAU's or clusters) in the disk boot sector, FAT, and

directory cannot be redirected either. Once again, the only method of

recovery during restoration errors for these FAUs is retry. User should be

informed of the seriousness of the problem in case of such a failure. This

is, in fact, a hard disk failure of the worst kind.  However, it may be

possible to change the partitioning of the disk being restored to in such a

manner as to place these non-redirectable sectors in a usable part of the

hard disk.  Normally this means making the partition start at a slightly

higher cylinder number and making the partition slightly smaller by the same

amount.

A saved disk image is only restorable to "compatible" disks, inspite of the

compression techniques. Because of the logical structure of the image (FAT

and directory and so on), it may only be restored to disks with the same: FAU

size in bytes, FAT size in bytes, directory size in bytes, and finally boot

block size in bytes.  However this does offer some leeway in terms of

slightly varying size disks with similar MS-DOS DPB parameters. Basically,

they may differ in size in the number of FAUs and only within small limits.

Assuming normal guidelines for MS-DOS BIOS implementors are usually followed,

it is likely but not certain that two disks of nominally the same size but

with different MS-DOS device drivers will be compatible by our definition.

Thus addons with different software may or may not be compatible by this

definition. Similarly hard disks for differing machines may or may not be

compatible by this definition. The program does check for this kind of

compatibility and rejects a restoration to an incompatible disk. Hard disks

of the same capacity from the same manufacturer will almost always be

compatible by this definition.  Partitions of compatible disks which are

approximately the same size will usually also be compatible by this

definition.

The entire physical disk may not be represented by the MS-DOS DPB parameters.

It may be divided into several logical disk drives. For example this is done

by the Z100. Thus it requires a separate backup program for each operating

system, and it requires backing up each logical drive separately.

------------------------------------------------------------------------------

It is suggested the MS-DOS program CHKDSK be run prior to using the IMAGE

copy program for an image backup. CHKDSK will test for some disk data

integrity problems and fix some of them, and possibly warn of others which

the user should fix prior to doing an image backup. It will also reclaim some

unnecessarily allocated space. Such space may be necessary during the

restoration of the disk image, should there be bad sectors on the disk

restored to. Such wasted space may also force the use of more than the

minimum number of tapes if it is not reclaimed.  For similar reasons it is

suggested that any unnecessary or temporary files which do not need backing

up be deleted.

The following commands are suggested:

CHKDSK d: /F		("d" is the drive, answer Y to questions)

DEL \*.CHK		(Delete reclaimed file space)

------------------------------------------------------------------------------

If files were lost because of fixed disk hardware problems such as new bad

sectors, it is suggested that the MS-DOS FORMAT program should be used to

reformat the fixed disk prior to using the IMAGE copy program do an image

restore. Although the IMAGE backup program is capable of finding and marking

the bad sectors on the fixed disk, the MS-DOS FORMAT program (or perhaps

another diagnostic program) may be able to cure them. This is not usually

necessary if files were lost because of accidental deleting or program error.

----------------------------

| Error and Info. Messages |--------------------------------------------------

----------------------------

There are several categories of messages displayed by IMAGE during a copy

operation. First of all, each message has a criticalness level indicated by

the beginning of the message. Below is a list of the message "beginnings".

""

Messages with no decernable prefix are usually just requesting user

action.

"*"

Messages beginning with a "*" are progress messages which indicate

how far along things are.

"Info:"

This is purely an informative message. They are used to explain what

is currently happening and why. They usually accompany "unusual" tape

movement, such as backspacing (pausing) or high speed track seeking.

These messages are never cause for concern. Such messages will only

be displayed when the appropriate program parameter is selected

before starting the copy. By default they will not be displayed.

"Warning:"

A physical block number is displayed and an indication of which

device had the problem. One try/retry attempt was made at the current

block and unless this is the last attempt allowed, another will be

attempted. This accompanies what is termed a "soft" error. Generally

these messages are no cause for concern. Such messages will only be

displayed when the appropriate program parameter is selected before

starting the copy. By default they will not be displayed.

"Block retry:"

A logical block number is displayed and an indication of which

device had the problem. One try/retry attempt was made at the current

block and unless this is the last attempt allowed, another will be

attempted. This accompanies what is termed a "soft" error. Generally

these messages are no cause for concern.  A warning message (which

may not have been diplayed because of parameter settings) which

preceeded this message may be of some assistance in further

explaining the problem.

"Block failure:"

A logical block number is displayed and an indication of which device

had the problem. Several retries at this block have been attempted

and no more attempts will be made. There is definitely a problem with

the current block which the program is trying to access. A block

retry message which preceeded this message may be of some

assistence in further explaining the problem. This accompanies what

is termed a "hard" error. The complete tape/disk is not yet

considered bad enough to reject. If it is writing, the program will

use a different block instead. So for writing, this message is

generally not a cause for much concern, although it means the

tape/disk is imperfect. If it is reading and ignoring disk errors

has been selected, it will continue as though that the block was

read correctly for the moment, but later indicate which files were

affected. So for reading, this message implies that some data will

be lost. It is not likely that retrying the entire copy operation

will recover this data, but if such an attempt is necessary, it is

suggested that the retry limits be increased.

"Volume failure:"

Means that the particular tape being used is causing the problems. It

is likely that replacing the tape with a new tape or possibly

reformatting the tape will cure the problem. A block failure message

which preceeded this message may be of some assistence in further

explaining the problem. The program will ask for a new tape to be

mounted.

"Image failure:"

Means that the copying process will be aborted completely. All

attempts at recovery have failed. It is not likely that retrying the

entire copy operation will succeed, but if such an attempt is

necessary, it is suggested that the retry limits be increased.

"Internal error detected by ... "

Probably a program bug detected by the program itself.

------------------------------------------------------------------------------

Normal messages:

"Insert new tape"

Program requires a new tape to be mounted. After the new tape is

mounted the program will continue without user keyboard input.

"* Starting volume 1"

The program is processing the numbered tape volume for the first

time.

"* Restarting volume 1"

The program is processing the numbered tape volume starting at the

beginning again. This follows a volume failure.

"* Image name is: MOTHERSDISK of 01/01/1980 00:00"

The name of the disk which was backed up is displayed as it was saved

on the tape along with the current time and date. Each tape will have

the same information (including time and date) stamped on it for a

multivolume backup, so that it may be verified that they belong

together.

"* Tape last used by IMAGE version #.# for:

"* MOTHERSDISK (1 of 1) at 00:00 on 01/01/1980"

Indicates tape was used before. If this information doesn't match

previous info during a restore, an error message will be displayed

and a new tape asked for. During a backup, you will be asked if you

want to overwrite the tape.

"Do you want to re-use this tape (Y/N) [N]?"

You have an option to overwrite the current tape by saying "Y" (yes),

or retaining the current tape's information and mounting a different

tape by saying "N" (no).

"* Tape was never used before"

Indicates unused tape. During a backup this is desireable. During a

restore a new tape will be asked for.

"* Total volumes will be 1"

Indicates the total number of tapes which will be used.

"** Starting pass 1"

A new pass is beginning.

"*** Starting copy phase"

During this phase, data is actually copied from the source device to

the object device.

"*** Starting verify phase"

During this phase, data is read from the object device in order to

verify that is was written correctly.

"***** Completely successful image copy!"

Image copy was successfully completed and all is well with the tapes

and the disk.

------------------------------------------------------------------------------

Error messages:

Note that each message below will have a criticalness level indicated at the

beginning of the message line, as discussed above. The prefixes are generally

not included below.

"MOTHERSDISK is not the same as original source tape name: FATHERSDISK"

The tape is from a different backup set for a different disk.

"01/01/1980 is not the same as original source tape date: 02/02/1982"

The tape is from a different backup set for a different date.

"00:00 is not the same as original source tape time: 19:27"

The tape is from a different backup set for a different time.

"***** Aborted image copy!"

A control-C was typed by the user to abort the copy operation.

"Bad block(s) in non-redirectable part of tape"

The bad block table written by the format program indicates that

there is at least one bad block in the part of the tape that IMAGE

must use. This tape is not usable as an IMAGE tape, although it may

be usable for other programs. Possibly curable by reformatting the

tape.

"Bad system sector(s) on object disk"

Fixed disk has bad sectors in the system area, FAT area, or root

directory area. Possibly curable by reformating the disk or by using

a different disk partitioning.

"Cannot read redirect table"

"Cannot write redirect table"

The program cannot read/write the table at the start of the tape

which indicates how logical blocks are redirected onto physical

blocks. Without this table, none of the rest of the tape can be

placed in the right place on the disk. Probably the tape is bad and

a different tape should be used.

"Cannot read bad block table"

The program cannot read the table at the start of the tape which

indicates which blocks were found at formatting time to be bad. Since

this block also has other useful information and is the required

place for the volume name information, the copy operation cannot

continue. Probably the tape is bad and a different tape should be

used.

"Cannot read header block"

The program cannot read the volume name information. This block also

has the bad block table in it. Probably the tape is bad and a

different tape should be used.

"Directory: \SOME\PATH\ANDDIRNAME contains bad cluster #: x#, run CHKDSK"

There is an internal inconsistency to the disk file allocation table

(FAT) in which a directory contains a cluster which was marked as

having bad sectors. If CHKDSK had been run prior to the backup of

the disk it would probably have reported this. The only solutions

are to use the /F parameter on CHKDSK to truncate the file or to

delete the file.

"Disk drive parameters are significantly different"

Data on tape is from a different kind or size fixed disk or fixed

disk partition, and cannot be placed on the fixed disk. Data should

be restored to the original kind of fixed disk or to (roughly) the

same size partition.

"***** Failed image copy!"

The image copy was aborted by the IMAGE program. Something went so

wrong that the program could not continue. Preceeded by more

informative explanitory messages.

"FAT chain starting at # ended on cluster w/bad ptr #, run CHKDSK"

There is an internal inconsistency to the disk file allocation table

(FAT) in which a file contains a cluster which was marked as having

bad sectors. This file will later be named in another warning

message. If CHKDSK had been run prior to the backup of the disk it

would probably have reported this. The only solutions are to use the

/F parameter on CHKDSK to truncate the file or to delete the file.

"File: \SOME\PATH\ANDNAME.EXT contains bad cluster #: x#, run CHKDSK"

There is an internal inconsistency to the disk file allocation table

(FAT) in which a file contains a cluster which was marked as having

bad sectors. If CHKDSK had been run prior to the backup of the disk

it would probably have reported this. The only solutions are to use

the /F parameter on CHKDSK to truncate the file or to delete the

file.

"File allocation chain has loop in file: \SOME\PATH\ANDNAME.EXT, run CHKDSK"

There is an internal inconsistency to the disk file allocation table

(FAT) which causes this file to loop back onto itself. If CHKDSK had

been run prior to the backup of the disk it would probably have

reported this. The only solutions are to use the /F parameter on

CHKDSK to truncate the file or to delete the file.

"File allocation chain has loop in directory: \SOME\PATH\ANDNAME, run CHKDSK"

There is an internal inconsistency to the disk file allocation table

(FAT) which causes this directory file to loop back onto itself. If

CHKDSK had been run prior to the backup of the disk it would probably

have reported this. The only solutions are to use the /F parameter on

CHKDSK to truncate the directory or to delete the files	in the

directory and then delete the directory.

"File not completely copied: \SOME\PATH\ANDNAME.EXT"

This file was uncopyable because of bad sectors on the fixed disk or

the tape.  If it is a directory name then additional files in that

directory are probably lost. Normally bad sectors on the disk are

simply redirected elsewhere, but the disk may be completely full.

"Not enough buffer space to read directory: \SOME\PATH\ANDDIRNAME"

Did not have enough memory to check the validity of a subdirectory

and redirect any first cluster (FAU) references in it. This directory

should be hand checked or more memory added and the copy redone. Most

likely, if the data is being restored to the same disk, nothing will

be wrong with anything in the directory. If anything is wrong, it

will be that files will have been shortened (bad sectors removed) or

invalid (directory first cluster not changed). CHKDSK may be used to

detect the latter.

"Not enough RAM for buffers required"

Means there is not enough memory for the image program. Detected when

attempting to allocate space for buffers. More memory to computer is

required or a reduction in the size of the operating system.

"Not enough space for queues"

Means there is not enough memory for the image program. Detected when

attempting to allocate space for queue of tape blocks to copy. More

memory to computer is required or a reduction in the size of the

operating system.

"***** Partially successful image copy!"

Some data was lost during the copy because of unreadable or

unwriteable sectors on the tape or fixed disk. Files not previously

mentioned in error messages should be OK.

"Redirection table overflow"

Too many hard errors on the tape. More blocks need redirection than

there is provision for in the spare block area. This may be retried

with a large spare block percentage, but it would be better to use a

less imperfect tape.

"Source tape major version (##) does not match program version (##)"

A different version of IMAGE wrote this tape. This message will only

be displayed if the versions are incompatible.

"Source tape minor version (##) does not match program version (##)"

A different version of IMAGE wrote this tape. This message will only

be displayed if the versions are incompatible.

"Source tape volume (##) does not match expected volume (##)"

Either this tape is not from the right set or it is not the next

volume. Usually just a problem with the order that the tapes were

mounted. Maybe they were incorrectly marked on the external label.

Try a different one from the same set.  The program will ask for

another	tape.

"Source tape volume total (##) does not match previous total (##)"

Probably a tape from a different backup set, although possibly an

internal program error.

"Tape already used in this backup"

Same tape was inserted twice for a multiple tape backup. Must make

sure tapes are correctly marked and a different tape is used for each

volume.

"Tape is not an IMAGE tape"

The tape wasn't written by IMAGE, so it cannot be restored by IMAGE

to the disk.

------------------------------------------------------------------------------

Warning messages:

"Disk error reading for physical sector #-(xxxxxxxx)"

"Disk error writing for physical sector #-(xxxxxxxx)"

The "xxxxxxxx" could be one of the following MS-DOS error messages.

0000    "Write protected"

Unlikely for a hard disk, unless it is removeable.

0001    "Unknown unit"

Unlikely.

0002    "Unit not ready"

Hard disk may need to be powered on, or inserted if removeable.

0003    "Unknown command"

Unlikely.

0004    "Data error"

Usually a bad disk sector.

0005    "Bad request"

Unlikely.

0006    "Seek error"

Usually a bad disk track.

0007    "Unknown media"

Unlikely.

0008    "Sector not found"

Usually a bad disk sector.

0009    "Out of paper"

Unlikely for a hard disk.

000a    "Write fault"

Usually a bad disk sector, track, or possibly disk read/write head.

000b    "Read fault"

Usually a bad disk sector, track, or possibly disk read/write head.

000c    "General failure"

Usually a bad disk sector.

In some cases it may ask you for a retry: "Retry (Y/N) [Y]?"

Answering "N" (no) will abort the copy operation. If your disk may be

appropriately adjusted (such as turning on the power supply), then

the retry may be successful when you answer with "Y" (yes). In other

cases it means part of the disk is bad, and the program will recover

by itself if it can. The physical sector number is the same one which

may be used to access the block with the problem when using MS-DOS

calls or utilities (like DEBUG). The read or write may be of several

consecutive sectors (not indicated in the msg), so the sector with

the problem may be numbered a few higher than the number in the

message. Sometimes using DEBUG to write that the bad sector will

eliminate the problem.

------------------------------------------------------------------------------

In the following tape error warning messages, any one of the error messages

below may appear in place of "xxxxxxxx":

0000    "still busy",

Waiting for not busy failed.

May be a major problem with the particular tape.

May be a problem with the drive.

0001    "not accepted",

Drive did not accept command.

Probably a problem with the drive or drive not on line.

0002    "read controller",

Problem with controller accepting command.

May be a problem with the floppy disk controller.

0003    "write controller",

Problem with controller responding to command.

May be a problem with the floppy disk controller.

0004    "controller error",

Invalid controller response.

May be a problem with the floppy disk controller.

0005    "record not found",

No valid ID read.

Possibly a problem with the particular tape block, although most

likely caused by slow simultaneous processing of the last disk block

because	of soft disk errors. If this happens more often than about

every 10 sectors, it may be preferable to select non-overlapping tape

and disk I/O.

0006    "sector CRC error",

Checksum error on record.

Probably a soft error on tape block.

Possibly a problem with the particular tape block.

0007    "DMA error",

DMA processor missed DRQ (data request), data lost.

Possibly a problem with the floppy disk controller.

Possibly a problem with interference with floppy disk software.

0008    "write protected",

Tape is write protected.

Try moving write protect switch on tape.

0009    "id not found",

No valid ID read.

Probably a problem with the particular tape block.

0010    "interrupt timeout",

I/O never properly completed.

0011    "DMA boundary"

Internal problem with program buffer boundary alignment.

????    "error code out of range"

Internal problem with program subroutine returning invalid return

code.

The "physical blk #" in the following messages indicates where the program

estimated (sometimes incorrectly) that the tape was positioned at the time of

the error. The block numbering runs throughout the tape for these messages,

from 1 to the last block, and appear in hex.

"Tape error during I/O for physical blk #-(xxxxxxxx)"

Program detected error while waiting for completion of I/O.

Usually just means a bad block.

"Tape error exitting servo for physical blk #-(xxxxxxxx)"

IMAGE is making sure that the tape drive is not in servo mode.

If it was already out of servo mode, this message results.

This message should just be ignored.

"Tape error initializing for physical blk #-(xxxxxxxx)"

Program could not initialize the drive.

"Tape error moving forward for physical blk #-(xxxxxxxx)"

Program could not get drive to start moving tape.

Usually because tape is at end of track already.

Possibly a major tape problem or a drive or controller problem.

"Tape error moving to end for physical blk #-(xxxxxxxx)"

Program could not get drive to move tape to end at high speed.

A major tape problem or a drive or controller problem.

"Tape error pausing for physical blk #-(xxxxxxxx)"

Program attempted to backspace (pause) tape, but drive did not

properly acknowledge it.

A major tape problem or a drive or controller problem.

"Tape error positioning for physical blk #-(xxxxxxxx)"

Program could not position tape appropriately to initiate I/O.

Usually just a bad block on tape.

"Tape error reading for physical blk #-(xxxxxxxx)"

Program detected error while initiating a read.

Usually major tape problem or a drive or controller problem.

"Tape error reseting for physical blk #-(xxxxxxxx)"

Program could not reset the drive firmware.

A major tape problem or a drive or controller problem.

"Tape error seeking loadpt for physical blk #-(xxxxxxxx)"

Program could not get drive to seek to load point.

A major tape problem or a drive or controller problem.

"Tape error seeking track for physical blk #-(xxxxxxxx)"

Program could not get drive to seek to track for the block.

A major tape problem or a drive or controller problem.

"Tape error sensing for physical blk #-(xxxxxxxx)"

Program detected bad sense data.

Generally a write protect error.

"Tape error stopping for physical blk #-(xxxxxxxx)"

Program could not get drive to stop moving the tape.

A major tape problem or a drive or controller problem.

"Tape error track following for physical blk #-(xxxxxxxx)"

Program detected failure of tape head to follow the right track.

Usually a momentary and not serious problem.

Possibly a major tape problem or a drive or controller problem.

"Tape error waiting for physical blk #-(xxxxxxxx)"

Program gave up on waiting for tape drive to become "not busy".

A major tape problem or a drive or controller problem.

"Tape error writing for physical blk #-(xxxxxxxx)"

Program detected error while initiating a write.

Usually a major tape problem or a drive or controller problem.

------------------------------------------------------------------------------

Information messages:

"No room for source disk cluster in relocation table"

The disk cluster is unwritable and would be redirected, but there is

not enough space in the relocation table to remember where all the

redirected clusters were moved. The associated file name will be

indicated later in an error message.

"Now estimated position at trk #, blk #, sec #; looking for blk #"

Since tape is not currently moving, program now knows that its

record of where the head is on the tape is only an estimate.

"Now examining directory: \SOME\PATH\ANDDIRNAME"

During the termination of a backup or restore operation, the program

is looking through the directory to match up bad block problems with

corresponding file names so that appropriate error messages may be

displayed.

"Now examining File Allocation Table (FAT)"

During the termination of a backup or restore operation, the program

is looking through the FAT to match up bad block problems with

corresponding file names so that appropriate error messages may be

displayed.

"Now examining root directory"

During the termination of a backup or restore operation, the program

is looking through the directory to match up bad block problems with

corresponding file names so that appropriate error messages may be

displayed.

"Now looking for FAT clusters in corrupted files"

During the termination of a backup or restore operation, the program

is following file chains backward through the FAT to find the start

of files with bad blocks, so that appropriate error messages may be

displayed.

"Now marking bad source FAT clusters as available"

During the termination of a restore operation, the program is marking

in the FAT all the bad spots on the disk so that they will not be

used as places to put files.

"Now modifying File Allocation Table (FAT)"

During the termination of a restore operation, the program is making

all necessary changes in the FAT which are required because of

redirection of bad sectors or not being able to redirect bad sectors.

"Now moving tape at trk #, blk #, sec #; looking for blk #"

Program will now try to get tape into motion.

"Now moving to track #"

Program moved tape head to given track.

"Now pausing at trk #, blk #, sec #; looking for blk #"

Program has backspaced (paused) the tape for purposes of tape

positioning prior to I/O.

"Now seeking track #"

Program is seeking the indicated track.

"Now reading id at trk #, blk #, sec #; looking for blk #"

Program is reading sector ids while waiting for the block

indicated to pass under the tape head.

"Now redirecting bad FAT clusters"

During the termination of a restore operation, the program is making

all necessary changes in the FAT which are required because of

redirection of bad sectors.

"Now removing bad FAT clusters from corrupted files"

During the termination of a restore operation, the program is making

all necessary changes in the FAT which are required because of not

being able to redirect bad sectors.

"Now reseeking at trk #, blk #, sec #; looking for blk #"

Program has moved to start of tape track at high speed for purposes

of tape positioning prior to I/O.

"Now reseeking track #"

Program is attempting to get on the indicated track again.

Usually done after an on track test failure.

"Now testing extremes at trk #, blk #, sec #; looking for blk #"

Program is checking whether the head is at an extreme (start or end)

of the tape.

"Relocating source disk cluster # to object #"

The first disk cluster is unwritable and is being redirected to

another available cluster.  The directory and file allocation table

will be updated later.

"Testing for online drive number #"

Program is testing the controller drive lines to find the tape drive.

"Total of # warnings and # errors reading source"

"Total of # warnings and # errors writing object"

"Total of # warnings and # errors verifying object"

Summary of errors of various kinds. An informative message. None is

cause for concern in the absence of more serious messages.

"Unable to relocate source disk cluster # on object"

The disk cluster is unwritable and would be redirected, but there is

no space to redirect it to. The associated file name will be

indicated later in an error message.

------------------------------------------------------------------------------

Internal error messages:

Internal errors indicate program bugs which may or may not be avoidable, but

do require fixing. They were detected as a result of internal consistency

checks to insure the quality of the software, and should never actually

happen, except possibly during debugging. Since they mean that there is a

problem with the software, it should be noted that they do not mean that

anything is wrong with your tapes or your drive or your controller. Not much

you can do about these messages as a user other than to contact Irwin

Magnetics to get a new copy of the IMAGE program.

"Internal error detected by queput: Block group table overflow"

Program ran out of allocated room while queuing blocks to copy.

"Internal error detected by rdrctbad: Block range error"

Block number is out of range.

"Internal error detected by rdrctadd: Block range error"

Block number is out of range.

"Internal error detected by dk2iohi: Block range error"

Block number is out of range.

##########################