HARD DISK DRIVE DATA:

LEGEND:

HDS = the number of data surfaces

CYL = the number of cylinders

RWC = reduced write current cylinder

WP = write precompensation cylinder

NOTES:

1. Wedge Servo - not usable on the IBM XT controller, and not

usable on revision "A" XEBEC 1210 and 1220 controllers.

2. Drive connector J1 pin 2 may need to be disabled when used

on the AT.

3. Removable media.

4. Cut jumper W3 before installing.

MODEL   :HDS: CYL : RWC : WP  : NOTES

--------+-----------+---+-----+-----+-----+--------

ATASI   :  AT3020   : 3 : 635 :  0  :  0  : Note 1

:  AT3033   : 5 : 635 :  0  :  0  : Note 1

:  AT3046   : 7 : 635 : 635 : 320 :

:  AT3051   : 7 : 704 : 704 : 350 :

:  AT3053   : 7 : 733 : 733 : 365 :

:  AT3085   : 8 :1024 :1024 : 512 :

--------+-----------+---+-----+-----+-----+--------

BULL    :  D530     : 3 : 987 : 987 : 987 :

:  D550     : 5 : 987 : 987 : 987 :

:  D570     : 7 : 987 : 987 : 987 :

:  D585     : 7 :1166 :1166 :1166 :

--------+-----------+---+-----+-----+-----+--------

CDC     : 9415-5-21 : 3 : 697 : 697 :  0  :

: 9415-5-36 : 5 : 697 : 697 :  0  :

: 9415-5-48 : 5 : 925 : 925 : 128 :

: 9415-5-67 : 7 : 925 : 925 : 128 :

: 9415-5-86 : 9 : 925 : 925 : 128 :

: 94205-30  : 3 : 989 : 989 :  0  :

: 94205-51  : 5 : 989 : 989 :  0  :

: 94155-25  : 4 : 615 : 615 : 300 :

: 94155-38  : 5 : 733 : 733 : 733 :

: 94155-48  : 5 : 925 : 925 :  0  : Note 2

: 94155-57  : 6 : 925 : 925 : 128 :

: 94155-67  : 7 : 925 : 925 :  0  : Note 2

: 94155-77  : 8 : 925 : 925 : 128 :

: 94155-85  : 8 : 925 :1024 :1024 :

: 94155-85P : 8 : 925 :1024 : 128 :

: 94155-86  : 9 : 925 : 925 :  0  :

: 94155-96  : 9 :1024 :1024 :1024 :

: 94155-96P : 9 :1024 :1024 : 128 :

--------+-----------+---+-----+-----+-----+--------

CDC     : 94155-120 : 8 : 960 : 960 : 960 :

cont.   : 94155-120P: 8 : 960 : 960 : 128 :

: 94155-135 : 9 : 960 : 960 : 960 :

: 94155-135P: 9 : 960 : 960 : 128 :

: 94156-48  : 5 : 925 : 925 : 925 :

: 94156-67  : 7 : 925 : 925 : 925 :

: 94156-86  : 9 : 925 : 925 : 925 :

: 94205-51  : 5 : 989 : 989 : 128 :

: 94166-101 : 5 : 969 : 969 : 969 : ESDI

: 94166-141 : 7 : 969 : 969 : 969 : ESDI

: 94166-182 : 9 : 969 : 969 : 969 : ESDI

: 94171-300 : 9 :1412 :1412 :1412 : ESDI

: 94171-344 : 9 :1549 :1549 :1549 : ESDI

: 94181-574 :15 :1224 :1224 :1224 : ESDI

: 94186-383 :13 :1412 :1412 :1412 : ESDI

: 94186-383H:15 :1412 :1412 :1412 : ESDI

: 94186-442 :14 :1412 :1412 :1412 : ESDI

: 94205-77  : 5 : 989 : 989 : 128 :

--------+-----------+---+-----+-----+-----+--------

CDC     : 94216-106 : 5 :1024 :1024 :1024 : ESDI

cont.   : 94335-55  : 5 : 625 : 625 : 128 :

: 94335-100 : 9 : 625 : 625 : 128 :

: 94335-150 : 9 : 938 : 938 : 128 :

: 94355-100 : 9 : 625 : 625 : 128 :

: 94355-55  : 5 : 625 : 625 : 128 :

: 94355-150 : 9 : 938 : 938 : 128 :

: 94356-200 : 9 :1250 :1250 :1250 : ESDI

--------+-----------+---+-----+-----+-----+--------

CMI     : CM3426    : 4 : 612 : 612 : 612 : Note 1

: CM5205    : 2 : 256 : 256 : 256 : Note 1

: CM5410    : 4 : 256 : 256 : 256 :

: CM5616    : 6 : 256 : 256 : 256 :

: CM6426    : 4 : 615 : 256 : 256 :

: CM6426S   : 4 : 640 : 256 : 256 :

: CM6640    : 6 : 640 : 640 : 256 :

: CM7660    : 6 : 960 : 960 : 512 :

: CM7880    : 8 : 960 : 960 : 512 :

--------+-----------+---+-----+-----+-----+--------

MODEL   :HDS: CYL : RWC : WP  : NOTES

--------+-----------+---+-----+-----+-----+--------

FUJI    : FK305-26  : 4 : 615 : 615 : 615 :

: FK305-26R : 4 : 615 : 615 : 615 :

: FK305-39  : 6 : 615 : 615 : 615 :

: FK305-39R : 6 : 615 : 615 : 615 :

--------+-----------+---+-----+-----+-----+--------

FUJITSU :  M2225-ALL: 4 : 615 : 615 : 615 :

:  M2226-ALL: 6 : 615 : 615 : 615 :

:  M2227-ALL: 8 : 615 : 615 : 615 :

:  M2230AS  : 2 : 320 : 320 : 320 :

:  M2233AS  : 4 : 320 : 320 : 320 :

:  M2234AS  : 6 : 320 : 320 : 320 :

:  M2235AS  : 8 : 320 : 320 : 320 :

:  M2230AT  : 2 : 320 : 320 : 320 :

:  M2233AT  : 4 : 320 : 320 : 320 :

:  M2241AS  : 4 : 754 : 754 : 754 :

:  M2242AS  : 7 : 754 : 754 : 754 :

:  M2243AS  :11 : 754 : 754 : 754 :

:  M2244E   : 5 : 823 : 823 : 823 : ESDI

:  M2245E   : 7 : 823 : 823 : 823 : ESDI

:  M2247E   : 7 :1243 :1243 :1243 : ESDI

:  M2246E   :10 : 823 : 823 : 823 : ESDI

:  M2249E   :15 :1223 :1223 :1223 : ESDI

--------+-----------+---+-----+-----+-----+--------

HITACHI :  DK511-3  : 5 : 699 : 256 : 256 :

:  DK511-5  : 7 : 699 : 256 : 256 :

:  DK511-8  :10 : 823 :  ?  :  ?  :

--------+-----------+---+-----+-----+-----+--------

MAXTOR  :  XT-1065  : 7 : 918 : 918 : 918 : Note 2

:  XT-1085  : 8 :1024 :1024 :1024 :

:  XT-1105  :11 : 918 : 918 : 918 :

:  XT-1120R : 8 :1024 :1024 :1024 :

:  XT-1140  :15 : 918 : 918 : 918 :

:  XT-1240  :15 :1024 :1024 :1024 :

:  XT-2085  : 7 :1224 :1224 :1224 : Note 2

:  XT-2140  :11 :1224 :1224 :1224 :

:  XT-2190  :15 :1224 :1224 :1224 :

: EXT-4175  : 7 :1224 :1224 :1224 : ESDI

: EXT-4280  :11 :1224 :1224 :1224 : ESDI

: EXT-4380  :15 :1224 :1224 :1224 : ESDI

: EXT-8380  : 8 :1632 :1632 :1632 : ESDI

: EXT-8760  :15 :1632 :1632 :1632 : ESDI

--------+-----------+---+-----+-----+-----+--------

MEMOREX :  323      : 6 : 320 : 320 : 128 :

:  324      : 8 : 320 : 320 : 128 :

:  512      : 3 : 961 : 961 : 480 :

:  513      : 5 : 961 : 961 : 480 :

:  514      : 7 : 961 : 961 : 480 :

--------+-----------+---+-----+-----+-----+--------

MICRO-  :  1302     : 3 : 830 : 830 : 400 :

POLIS   :  1303     : 5 : 830 : 830 : 400 :

:  1304     : 6 : 830 : 830 : 400 :

:  1323     : 4 :1024 :1024 :1024 :

:  1323A    : 5 :1024 :1024 :1024 :

:  1324     : 6 :1024 :1024 :1024 :

:  1324A    : 7 :1024 :1024 :1024 :

:  1325     : 8 :1024 :1024 :1024 :

:  1333A    : 5 :1024 :1024 :1024 :

:  1334     : 6 :1024 :1024 :1024 :

:  1335     : 8 :1024 :1024 :1024 :

:  1353     : 4 :1024 :1024 :1024 : ESDI

:  1353A    : 5 :1024 :1024 :1024 : ESDI

:  1354     : 6 :1024 :1024 :1024 : ESDI

:  1354A    : 7 :1024 :1024 :1024 : ESDI

:  1355     : 8 :1024 :1024 :1024 : ESDI

:  1554     :11 :1224 :1224 :1224 : ESDI

:  1555     :12 :1224 :1224 :1224 : ESDI

--------+-----------+---+-----+-----+-----+--------

MICRO-  :  1556     :13 :1224 :1224 :1224 : ESDI

POLIS   :  1557     :14 :1224 :1224 :1224 : ESDI

cont.   :  1558     :15 :1224 :1224 :1224 : ESDI

:  1566-11  :11 :1632 :1632 :1632 : ESDI

:  1567-12  :12 :1632 :1632 :1632 : ESDI

:  1567-13  :13 :1632 :1632 :1632 : ESDI

:  1568-15  :14 :1632 :1632 :1632 : ESDI

:  1568-15  :15 :1632 :1632 :1632 : ESDI

:  1653-4   : 4 :1249 :1249 :1249 : ESDI

:  1653-5   : 5 :1249 :1249 :1249 : ESDI

:  1653-6   : 6 :1249 :1249 :1249 : ESDI

:  1653-7   : 7 :1249 :1249 :1249 : ESDI

--------+-----------+---+-----+-----+-----+--------

MICRO-  :  HH312    : 4 : 306 : 306 : 306 :

SCIENCE :  HH325    : 4 : 612 : 612 : 612 :

:  HH330    : 4 : 612 : 612 : 612 :

:  HH612    : 4 : 306 : 306 : 306 :

:  HH725    : 4 : 612 : 612 : 612 :

:  HH825    : 4 : 612 : 612 : 612 :

:  HH830    : 4 : 612 : 612 : 612 :

:  HH1050   : 5 :1024 :1024 :1024 :

:  HH1060   : 5 :1024 :1024 :1024 :

:  HH1075   : 7 :1024 :1024 :1024 :

:  HH1090   : 7 :1314 :1314 :1314 :

:  HH1095   : 7 :1024 :1024 :1024 :

:  HH1120   : 7 :1314 :1314 :1314 :

:  HH2120   : 7 :1024 :1024 :1024 : ESDI

--------+-----------+---+-----+-----+-----+--------

MODEL   :HDS: CYL : RWC : WP  : NOTES

--------+-----------+---+-----+-----+-----+--------

MINI-   :  MS1012   : 4 : 306 : 154 :  0  :

SCRIBE  :  MS2012   : 4 : 306 : 306 :  0  :

:  MS3053   : 5 :1024 :1024 : 512 :

:  MS3412   : 4 : 306 : 306 : 128 :

:  MS3425   : 4 : 615 : 615 : 128 :

:  MS3438   : 4 : 615 : 615 : 128 :

:  MS3650   : 6 : 809 : 809 : 128 :

:  MS4020   : 4 : 480 : 480 :  0  :

:  MS6032   : 3 :1024 :1024 : 512 :

:  MS6053   : 5 :1024 :1024 : 512 :

:  MS6074   : 7 :1024 :1024 : 512 :

:  MS6079   : 5 :1024 :1024 : 512 :

:  MS6085   : 8 :1024 :1024 : 512 :

:  MS6128   : 5 :1024 :1024 : 512 :

:  MS8425   : 4 : 615 : 615 : 128 :

:  MS8438   : 4 : 615 : 615 : 128 :

:  MS9230   : 9 :1224 :1224 :1224 : ESDI

:  MS9380   :15 :1224 :1224 :1224 : ESDI

--------+-----------+---+-----+-----+-----+--------

NEC     :  5124     : 4 : 310 : 310 : 128 :

:  5126     : 4 : 612 : 612 : 128 : Note 1

:  5146     : 8 : 615 : 615 : 128 : Note 1

:  D5452    :10 : 823 : 823 : 128 :

:  D5652    :10 : 823 : 823 : 823 : ESDI

:  D3126    : 4 : 615 : 823 : 256 :

--------+-----------+---+-----+-----+-----+--------

NEWBURY : PENNY 340 : 8 : 615 : 615 : 615 : Note 3

:  XT-1065  : 7 : 918 : 918 : 918 : Note 2

:  XT-1085  : 8 :1024 :1024 :1024 :

:  XT-1105  :11 : 918 : 918 : 918 :

:  XT-1140  :15 : 918 : 918 : 918 :

:  XT-2085  : 7 :1224 :1224 :1224 : Note 2

:  XT-2140  :11 :1224 :1224 :1224 :

:  XT-2190  :15 :1224 :1224 :1224 :

--------+-----------+---+-----+-----+-----+--------

PRIAM/  : V130      : 3 : 987 : 987 : 987 :

VERTEX  : V150/ID40 : 5 : 987 : 987 : 987 :

: V170/ID60 : 7 : 987 : 987 : 987 :

: V185      : 7 :1166 :1166 :1166 :

: ID-45H    : 5 :1018 :1018 :1018 :

: ID/ED-75  : 5 :1156 :1156 :1156 :

: ID/ED-100 : 7 :1156 :1156 :1156 :

: ID/ED-130 :15 :1024 :1024 :1024 :

: ID/ED-230 :15 :1218 :1218 :1218 :

: 514       :11 :1224 :1224 :1224 :

: 519       :15 :1224 :1224 :1224 :

: 617       : 7 :1224 :1224 :1224 : ESDI

: 628       :11 :1224 :1224 :1224 : ESDI

: 638       :15 :1224 :1224 :1224 : ESDI

--------+-----------+---+-----+-----+-----+--------

QUANTUM :  Q520     : 4 : 512 : 256 : 256 : Note 1

:  Q530     : 6 : 512 : 256 : 256 : Note 1

:  Q540     : 8 : 512 : 256 : 256 : Note 1

--------+-----------+---+-----+-----+-----+--------

RODIME  :  RO102    : 4 : 192 :  96 :  0  :

:  RO103    : 6 : 192 :  96 :  0  :

:  RO104    : 8 : 192 :  96 :  0  :

:  RO202    : 4 : 320 : 132 :  0  :

:  RO203    : 6 : 320 : 132 :  0  :

:  RO204    : 8 : 320 : 132 :  0  :

:  RO202E   : 4 : 640 : 640 :  0  :

:  RO203E   : 6 : 640 : 640 :  0  :

:  RO204E   : 8 : 640 : 640 :  0  :

:  RO3045   : 5 : 872 : 650 :  0  :

:  RO3055   : 6 : 872 : 650 :  0  :

:  RO3065   : 7 : 872 : 650 :  0  :

:  RO5090   : 7 :1224 :1224 :  0  :

:  RO5130R  : 7 :1224 :1224 :  0  :

--------+-----------+---+-----+-----+-----+--------

RMS     :  RMS506   : 4 : 153 :  77 :  77 :

:  RMS512   : 8 : 153 :  77 :  77 :

--------+-----------+---+-----+-----+-----+--------

MODEL   :HDS: CYL : RWC : WP  : NOTES

--------+-----------+---+-----+-----+-----+--------

SEAGATE :  ST225    : 4 : 615 : 615 : 300 :

:  ST238    : 4 : 615 : 615 : 300 :

:  ST251    : 6 : 820 : 820 : 820 :

:  ST277    : 6 : 820 : 820 : 820 :

:  ST412    : 4 : 306 : 128 : 128 :

:  ST419    : 6 : 306 : 128 : 128 :

:  ST506    : 4 : 153 : 128 : 128 :

:  ST4026   : 4 : 615 : 615 : 615 :

:  ST4038   : 5 : 733 : 733 : 300 :

:  ST4038M  : 5 : 733 : 733 : 733 :

:  ST4051   : 5 : 977 : 977 : 977 :

:  ST4053   : 5 :1024 :1024 :1024 :

:  ST4077   : 5 :1024 :1024 :1024 :

:  ST4096   : 9 :1024 :1024 :1024 :

:  ST4144   : 9 :1024 :1024 :1024 :

:  ST4192   : 8 :1147 :1147 :1147 :

--------+-----------+---+-----+-----+-----+--------

SHUGART :  SA604    : 4 : 160 : 128 : 128 :

:  SA606    : 6 : 160 : 128 : 128 :

:  SA612    : 4 : 306 : 128 : 128 :

:  SA712    : 4 : 320 : 128 : 128 :

--------+-----------+---+-----+-----+-----+--------

SYQUEST :  SQ306RD  : 2 : 306 : 306 : 306 : Note 3

:  SQ312RD  : 2 : 615 : 615 : 615 : Note 4

:  SQ325F   : 4 : 612 : 612 : 612 :

:  SQ338F   : 6 : 612 : 612 : 612 :

--------+-----------+---+-----+-----+-----+--------

TANDON  :  TM252    : 4 : 306 : 306 : 306 :

:  TM262    : 4 : 615 : 615 : 615 :

:  TM362    : 4 : 615 : 615 : 615 :

:  TAN501   : 2 : 306 : 128 : 153 :

:  TAN502   : 4 : 306 : 128 : 153 :

:  TAN503   : 6 : 306 : 128 : 153 :

:  TM602S   : 4 : 153 : 128 : 153 :

:  TM603S   : 6 : 153 : 128 : 153 :

:  TM603SE  : 6 : 230 : 128 : 128 :

:  TM702AT  : 4 : 615 : 615 : 615 :

:  TM703    : 5 : 695 : 695 : 695 :

:  TM703AT  : 5 : 733 : 733 : 733 :

:  TM705    : 5 : 962 : 962 : 962 :

:  TM755    : 5 : 981 : 981 : 981 :

--------+-----------+---+-----+-----+-----+--------

TOSHIBA :  MK53FA/B : 5 : 830 : 830 : 512 : Note 2

:  MK54FA/B : 7 : 830 : 830 : 512 : Note 2

:  MK56FA/B :10 : 830 : 830 : 512 :

--------+-----------+---+-----+-----+-----+--------

TULIN   :  TL226    : 4 : 640 : 640 : 640 : Note 1

:  TL326    : 4 : 640 : 640 : 640 : Note 1

:  TL240    : 6 : 640 : 640 : 640 : Note 1

:  TL340    : 6 : 640 : 640 : 640 : Note 1

--------+-----------+---+-----+-----+-----+--------