;========================================================================

;

;	Rules Concerning the Generation of Sector Translate Tables,

;   Format Specifications, DBP's and DPH's for 5-1/4 inch Diskettes.

;

;

;    ACTTYP  (active type) is a control byte associated with every DPB

;	(Disk Parameter Block), which, while not a formal part of it is still

;	necessary for correct operation of the BIOS.  Each of these is

;	generated at Assembly, and identifies the physical type of drive/media

;	pairing associated with each logical parameter block.

;

;

;	Diskette / Drive Identifier Byte:

;

;	.....| Drive Bits | Media Bits |

;	      \           |           /

; Bit:	7   6	5   4	3   2	1   0

;

;	|   |	|   |	|   |	|   |_	Two Sided  (Format found was 2 sided)

;	|   |	|   |	|   |	|

;	|   |	|   |	|   |___|_____	"N" type format number:	    Sector:

;	|   |	|   |	|			0 = Single Density, 128 byte

;	|   |	|   |	|			1 = Double Density, 256 byte

;	|   |	|   |	|			2 = Double Density, 512 byte

;	|   |	|   |	|			3 = Double Density,  1K byte

;	|   |	|   |	|

;	|   |	|   |	|_____________	1 = Double, 0 = Single sided host drive

;	|   |	|   |

;	|   |	|   |_________________	1 = 80 track, 0 = 40 track host drive

;	|   |	|

;	|   |	|_____________________	Minifloppy ID (5-1/4 inch diskette)

;	|   |

;	|___|_________________________	Hard Disk and Memory Drive type ID's

;

;

;	Bits 0 - 2 above are determined from the media itself for 8 inch

;  diskettes -- for 5-1/4 inch diskettes the "N" format number is read

;  from the media, but since there is no "sidedness" flag on there media

;  this is determined by the format byte taken from the sector translation

;  table corresponding to that format number ("N" again).

;

;	For minifloppies, the Drive's sidedness and track density are included

;  during system generation to permit the runtime code to "adjust" the drive

;  to the media encountered.  This is done by double stepping 80 track drives

;  to correctly read and write 40 track formats.  If the drive is double sided,

;  the code will set the "Force Two Sided" control bit when necessary to read

;  a double sided disk, and otherwise leave the drive in single sided mode.

;

;	The upper 2 bits identify hard media and memory type drives, and

;  cannot be used (they must be set to zero for floppy disk drives).

;

;	. . . . .

;

;

;    FORMAT  SPECIFIER  Byte (algorythm control):

;

;	One of these is associated with every type of media accepted.

;	It's contents identify the track density (48 or 96 tpi), number

;	of sides (1 or 2), and track / head / sector algorythm used to

;	correctly locate the data placed on the diskette.

;

;

; Bit:	7   6	5   4	3   2	1   0

;

;	|   |	|   |	|   |	|   |_	Force Two Sided

;	|   |	|   |	|   |	|     ( Disk 1A controller hardware setup bit )

;	|   |	|   |	|   |	|

;	|   |	|   |	|   |	|_____  Track Density

;	|   |	|   |	|   |		0 = 48 tpi format (40 track)

;	|   |	|   |	|   |		1 = 96 tpi format (80 track)

;	|   |	|   |	|   |

;	|   |	|___|___|___|_________	2 ** 4 = 16 Possible Track, Head

;	|   |				algorythms (see below)

;	|   |

;	|   |

;	|___|_________________________	Reserved control bits.

;

;   Legitimate bit patterns in Format ID byte:

;

;					   Trks |  Sides | Hd/Trk algorythm

;	x   x	0   0	0   0	0   0	 0 = 40   single	1 for 1

;	x   x	0   0	0   0	1   0	 1 = 80   single	1 for 1

;	x   x	0   0	0   1	0   1	 2 = 40   double   head = odd/even trk

;	x   x	0   0	0   1	1   1	 3 = 80   double   head = odd/even trk

;	x   x	0   0	1   0	0   1	 4 = 40   double   foldback at track 40

;	x   x	0   0	1   0	1   1	 5 = 80   double   foldback at track 80

;	x   x	0   0	1   1	0   1	 6 = 40   double   side 1, trk 0 -> 40

;	x   x	0   0	1   1	1   1	 7 = 80   double   side 1, trk 0 -> 80

;

;

;	This byte can be found in the sector translation table for 5-1/4 inch

;  disk drives.  Each header entry consists of 8 bytes:

;

;	0, 1 =	Word pointer to the actual sector translation table

;	   2 =	Length of actual sector translation table storage available

;	   3 =	Directory track (used only if directory buffering active)

;	   4 =	Gap 1 format size (disk read/write operation)

;	   5 =	Gap 2 format size (disk format creation)

;	   6 =	EOT, the value of the last physical sector on a track

;	   7 =	Format Specifier (or algorythm control) byte

;

;	There are a total of four header entries corresponding to each of the

;  possible 4 sector formats supported:    128, 256, 512, and 1024 bytes

;  per physical sector.  The Format Specifier byte controls how the drive is

;  manipulated in order to seek, read and write various diskette formats.

;

;  Note that of the existing 5-1/4 inch formats, a majority can be handled

;  by this software scheme / hardware controller pair, and are treated entirely

;  as native formats once the specifiers have been correctly set up.

;

;	The byte which identifies the directory track for each format is not

;  needed if you do not have "directory buffering" enabled in the BIOS.

;  When enabled, there is a significant speed-up in most data interchange

;  operations, particularly when the specified directory track correctly

;  identify the track on which the diskette's directory is located.

;

;	The Gap 1 and 2 sizes are used both for reading previously formated

;  diskettes and for formating fresh media to match the desired target machine.

;  The End Of Track (final physical sector) is also needed to correctly read

;  and write each format (full track buffering is always active).  Finally,

;  the sector translation table pointed to in the header must match the

;  original diskette's, and extra room has been left for this purpose.

;

;	. . . .

;

; 1. formatting using the gaps

; 2.

;

;	. . . . .

;

;	Physical Cylinder Seek:

;		(algorythm control comes from 2 separate bytes)

;

;	    ACTTYP bit 4

;			   ID bit #1	   Drive (host) : Format (diskette)

;		    0		0	0  =   40 track | 40 track

;		    0		1	1  =   40 track | 80 track (illegal)

;		    1		0	2  =   80 track | 40 track

;		    1		1	3  =   80 track | 80 track

;

;

;	. . . . .

;

;	Please examine the examples in the BIOS carefully before significantly

;  modifying the code presented.  There are a large number of traps involving

;  the correct generation of Disk Parameter Blocks, Allocation and Checksum

;  Vectors; it is quite possible to destroy your system disk in the process

;  of discovering the correct formulation.  Otherwise enjoy.		   aep.

;

;=====================================================================

########################################################