If you are not using NE2000 cards you must use the packet driver supplied

by the manufacturer.  Here are some examples of cards we have worked with.

SMC EtherCard PLUS Elite16

--------------------------

Sample AUTOEXEC.BAT:

\lbl\8003PKDR /r:D000  /b:280  /i:3

\lbl\net8

\lbl\utils\PKT_LINK debug

\lbl\net21

\lbl\utils\ETHNAME eth:290 node:3

The packet driver SMC supplies is called 8003PKDR.EXE and

is found on their disk in the PKT_DVR subdirectory.

Copy this file onto your hard drive's subdirectory LBL.

Everex SpeedLink 8 bit:

-----------------------

\lbl\ftpslink 0x60 3 0x320

\lbl\utils\ethname node:3 eth:320 evx

\lbl\net8

\lbl\utils\pkt_link debug

\lbl\net21

3COM 3c501:

-----------

\lbl\3c501 0x60 3

\lbl\net8

\lbl\utils\pkt_link debug

\lbl\net21

3COM EtherLink III  (3c509):

----------------------------

\lbl\3c5x9pd 0x60

\lbl\net8

\lbl\utils\pkt_link debug

\lbl\net21

Intel EtherExpress:

-------------------

\lbl\exp16 0x60 0x300

\lbl\net8

\lbl\utils\pkt_link debug

\lbl\net21

Run SOFTSET supplied by Intel and specify the connector type.