(

T A R G E T   A S S O C I A T E S

J I B   R A Y   F O R T H

)

( Examples of Forth Assembler:)

BASE @ HEX                         ( save previous base)

VARIABLE VAR   1234 VAR !     ( define a variable and initialize it)

5678 CONSTANT CON                  ( define a constant)

(

1. Incrementing a variable - no parameters passed

)

CODE VAR+1

VAR LHLD,                     ( get 16 bit number pointed to into HL)

H INX,                        ( increment it)

VAR SHLD,                     ( put it back in memory)

NEXT                          ( prepare to get back to Forth)

END-CODE

(

2. Setting a variable equal to a constant - no parameters passed

)

CODE VAR=CON

CON H LXI,                    ( get constant value into HL)

VAR SHLD,                     ( store it in variable)

NEXT

END-CODE

(

3. Passing one parameter ( on stack) from Forth to assembly language

)

CODE INIT-VAR  ( n - )             ( initialize a variable)

H POP,                        ( get number from stack into HL)

VAR SHLD,                     ( store it in memory)

NEXT

END-CODE

(

4. Passing one parameter from assembly language to Forth ( onto stack)

)

CODE VAR@      ( - n )             ( get the value of a variable)

VAR LHLD,                     ( get value from memory location)

H-PUSH                        ( put it on stack)

END-CODE

(�5. Passing one parameter to/from Forth/assembly language

)

CODE /256      ( n - n/256 )       ( divide number by 256)

H POP,                        ( get number off stack)

H L MOV,                      ( move high byte into low)

0 H MVI,                      ( put a zero in high byte)

H-PUSH                        ( back on stack)

END-CODE

(

6�� Passin� tw� parameter� fro� Fort� t� assembl� languag� an� on� bac� fro� �

assembly language to Forth.

)

CODE ADD       ( n1 n2 - n3 )      ( n3 = n1 + n2)

H POP,                        ( get top number n2)

D POP,                        ( get other number n1)

D DAD,                        ( add DE to HL, result in HL)

H-PUSH                        ( put result on stack)

END-CODE

(

7�� Passin� on� paramete� fro� Fort� t� assembl� languag� an� tw� bac� fro� �

assembly language to Forth.

)

CODE 256/MOD   ( n1 - n2 n3 )      ( n2 = n1/256 n3 = n1 256 MOD)

H POP,                        ( get n1)

H E MOV,                      ( move high byte of n1 to low byte DE)

0 D MVI,                      ( make high byte of DE = 0)

0 H MVI,                      ( same for HL)

D-PUSH                        ( put DE on stack, then HL)

END-CODE

(

8. Using BEGIN UNTIL construct.

)

CODE DELAY     ( - )               ( delay for time dependent on CON)

CON H LXI,                    ( get value of CON)

BEGIN,    H DCX,              ( decrement HL)

L A MOV,

H ORA,              ( is HL = 0 ?)

Z                   ( conditional for UNTIL,)

UNTIL,

NEXT

END-CODE

(

9. Using IF ELSE THEN construct.

)

CODE ?VAR=CON  ( - f )        ( does VAR variable = constant CON ?)

VAR LHLD,                ( get variable value)�     CON D LXI,               ( get constant value)

L A MOV,

E XRA,                   ( compare low order bytes)

A E MOV,                 ( save result in E)

H A MOV,

D XRA,                   ( now compare high order bytes)

E ORA,                   ( include other result)

NZ                       ( conditional for IF,)

IF,       0 H LXI,       ( make result false)

ELSE,     -1 H LXI,      ( make result true)

THEN,

H-PUSH                   ( put result on stack)

END-CODE

(

10. Using BEGIN WHILE REPEAT construct.

)

CODE STALL     ( - )          ( delay for time dependent on VAR variable)

VAR LHLD,                ( get variable value)

BEGIN,    L A MOV,

H ORA,         ( see if HL=0)

NZ             ( conditional for WHILE,)

WHILE,    H DCX,         ( decrement if not zero yet)

REPEAT,

NEXT

END-CODE

BASE !                        ( restore original base)

END-OF-FILE

###########################################################################################################