INSTALLING MAGIC ON THE X WINDOW SYSTEM V. 11

(second release)

Marco Papa

USC Advanced Computing Support Center

Sept. 16, 1988

This installation instructions are a slightly modified version of the

installation of magic for the X Window System, version 10.4, written

by Doug Pan of Stanford University.

This distribution contains files necessary for modifying the Magic VLSI

layout system (version 4.7) for use with a workstation running the

X Window System, version 11.  This file describes how to make the

installation.

SOFTWARE REQUIREMENTS

The X Window System, Version 11, Release 1 or a more recent version.

HARDWARE REQUIREMENTS

Workstation with a monochrome or an 4, 6, or 8 bit plane color display.

Caution: compiling Magic by using the provided makefiles winds up taking

about 20 Meg of filespace.  You can reduce this amount by at least

8 Meg if you comment out the references to lint and installed files

(all copies to ~cad/src/magic/lib/lint and ~cad/src/magic/installed).

The executable version for the RT PC is slightly less than 900 kilobytes.

Other versions will differ.

DIFFERENCES IN USAGE

Unlike on an AED, where there is a separate terminal for text input and

output, when running Magic on the X Window System, you run Magic from

the workstation console.  The xterm from which you start magic will provide

text output during execution, while an X window created by magic serves

as the graphics display. Do not use the -d -r -g arguments when running

Magic on the X Window System.

In order for the mouse buttons to function correctly in Magic, comment

out or remap all entries in your .uwmrc under "Mouse buttons/key maps"

that use only a single button with no key combination.

INSTALLATION

The following steps are provided as a guide for installation. They have

been tested on IBM RT PCs and IBM 6152s.  If you encounter difficulties

in the process it may be caused by erroneous instructions.

0) Backup the content of your current ~cad/src/magic/graphics and

~cad/src/magic/windows directories and cd to ~cad/src/magic.

1) Copy the following files to the directory ~cad/src/magic/graphics:

grX1.c

grX2.c

grX3.c

grX4.c

grX5.c

grXInt.h

grX11.h

grMain.c

grCMap.c

grClip.c

grDStyle.c

grGlyphs.c

grLock.c

grNull.c

grText.c

graphics.h

graphicsInt.h

glyphs.h

X11helper.c

Makefile

2) Modify the following makefiles:

~cad/src/magic/magic/Makefile

- change "ALLLIBS= -lm" to "ALLLIBS= -lm -lX"

~cad/src/magic/utils/Makefile

- add -DBSD4_3 to ifdef out references to cleanup

in utils/flsbuf.c (the ifdefs are already in place

in the c file)

Optional steps 3 - 5 add the command addpath:

3) Copy windClient.c to ~cad/src/magic/windows/windClient.c

(will overwrite the older file).

4) Copy windCmdX.c to ~cad/src/magic/windows (this is a new file).

5) Modify ~cad/src/magic/windows/Makefile so that the file windCmdX.c

will be included in the compilation of the windows module. Add -DX

to the CFLAGS option.

You are now ready for compilation. For this and the following steps, you

must use csh.

6) In the root directory (~cad/src/magic) type the command "make magic"

This typically can take around 1.5 hours, depending on the machine,

so you might want to make a log file and come back later by using

"make magic >&! make.log &".

7) Move or copy ~cad/src/magic/magic/magic to ~cad/bin.

8) Move or copy ~cad/src/magic/graphics/X11helper to ~cad/bin.

9) Make sure that ~cad/bin is included in every Magic user's search path.

Before you can run Magic, you need to set up the ~cad/lib directory.

10) Copy ther file "displays" to ~cad/lib/displays.  The following is the

contents of the displays file provided with this distribution, and

included for your reference.  Your configuration might be different.

/dev/ttyp0 console std XWIND

/dev/ttyp1 console std XWIND

/dev/ttyp2 console std XWIND

/dev/ttyp3 console std XWIND

/dev/ttyp4 console std XWIND

/dev/ttyp5 console std XWIND

/dev/ttyp6 console std XWIND

/dev/ttyp7 console std XWIND

Note: All the pn's are possible pseudo ttys for the xterms. We assumed

that there would be a maximum of 8 xterms at any given time.

1) Copy colormap, glyphs, and dstyle files from ~cad/src/magic/cadlib

to ~cad/lib/magic/sys. We use the following files, you might need more:

.magic

bw.glyphs

color.glyphs

cmos.tech20

mos.7bit.dstyle5

mos.7bit.std.cmap1

windows7.glyphs

You have now completed the installation of Magic for the X Window System

version 11. Start magic by typing "magic" on your workstation keyboard.

Note: Since the Magic X window occupies most of the screen, you should move

your startup xterm to the bottom of the screen before starting Magic.

XDEFAULTS

This second release of magic for X-11 provides support for user preferences

set in $HOME/.Xdefaults.  The program name is "magic", and the option

names are:

geometry/Geometry		-- magic window geometry

borderwidth/borderWidth	-- width of the border of magic window

small/Small	 		-- small font name

medium/Medium 		-- medium font name

large/Large  		-- large font name

xlarge/Xlarge 		-- extra large font name

For example:

magic.geometry:		=500x500+0+0

magic.borderWidth:	2

magic.small:		vg-13

magic.medium:		fg-18

magic.large:		vrb-25

magic.xlarge:		vrb-37

FILES

graphics:

grX1.c grX2.c grX3.c grX4.c grX5.c grXInt.h grX11.h X11helper.c

grMain.c grCMap.c grClip.c grDStyle.c grGlyphs.c grLock.c

grNull.c grText.c graphics.h graphicsInt.h glyphs.h Makefile

windows:

windClient.c windCmdX.c

BUGS

Please report any problems with installation or use of Magic for the

X Window System, version 11 to:

Dr. Marco Papa

Advanced Computing Support Center

University of Southern California

3580 Wilshire Blvd., Ste 1910

Los Angeles, CA 90010

(213)743-3752

ARPA: papa@uscacsc.usc.edu

UUCP: ...!oberon!pollux!papa